NATIONAL INSTITUTE OF PLANT GENOME RESEARCH

Aruna Asaf Ali Marg, Near ICSSR, JNU Campus, New Delhi – 110067

Phone: 26735161, 26735138 Fax: 26741658

F. No. NIPGR/7/15 /							   	     Dated: 18.03.2013

##### NOTICE INVITING QUOTATIONS

Sealed item rate quotations are invited on behalf of the Director, NIPGR, New Delhi, from approved & eligible contractors in respect of the following work, so as to reach his office on or before 03.04.2013 up to 3.00 P.M.  The quotation shall be opened on the same day at 3.30 P.M. in presence of the intending quotationers.

SCHEDULE OF QUANTITY

Name of Work:	Supply & Installation of “Naturally Ventilated Poly House” at NIPGR 				Campus, New Delhi.

| Sl. No. | Description | Qty. | UNIT | Rate (in `) | Amount (in `) |
| --- | --- | --- | --- | --- | --- |
|  | As per enclosed Schedule of Quantities and Terms & Conditions |  |  |  |  |

Encl.:	1. Schedule of quantity, Technical specifications & Drawing.

2. Terms & conditions.

Administrative Officer

NIPGR

M/s ____________________________

_______________________________

_______________________________

__________________________________

Seal & Signature of Contractor

Schedule of Quantity

Name of Work: Supply & Installation of “Naturally Ventilated Poly House” at NIPGR 					Campus New Delhi.

| S.N. | Description | Unit | QTY | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Construction of Naturally Ventilated poly House of Size: 30ft x 20ft with Working Floor Area- 600 Sq.ft. Centre Height – 18’ Gutter Height – 12’ Shape: Saw tooth type As per enclosed Technical specification & drawing of Poly house. | Each | 1 |  |  |
|  |  | Total Amount |  |  |  |

(In words Rs.---------------------------------------

Administrative Officer

NIPGR

Seal & Sign Of Contractor

Technical specifications:

Infrastructure & Super Structure:  GI (47mm x 47mm/ 38mm x 38mm/ 25mm x 25mm  with  2mm thick ) Hot dip galvanized ISI standard Pipe IS 1239 /IS 1165 class B anti-corrosive, humidity resistant  with IS 4736-1968/ISO 65-1973  zinc coat as approved . For the maximum Indian wind load conditions120 km / hr, structural trusses with runners for absolute strength will be provided.

Cladding Material: High quality UV stabilized 5 Layered (Imported {Ginegar Make Israel}) LDPE Sheet 200 micron thick with Anti dust, anti condensed and anti drip properties. On all four sides polyfilm will be embedded inside the ground.

Poly Locks: Poly lock profile will be made up of Aluminium section fixed with zig -zag spring of GI material with 12 gauge to withstand the maximum 120 Km/hrs wind load.

Door:	 (Door size : 6’3” x3’) of GI pipe section with LDPE sheet 200 micron thick  as cladding material with lock and pinion system.

Curtain: The curtain pipes will be provided to control two side ventilation on length wise. The curtain will be provided with manual gear operated opening system.

Ventilation: Two side ventilation up to the height of 10’ & 60cm top ventilation with 40 x 40 nylon mess will be provided.

Side Wall Foundation:

CC foundation (1:3:6) 1’ below and 1.5’ above ground level to checks lateral moments of water, Base PCC 2”thick , than 13” wide  first base line , than 9”wide second base line, Damp proof cement (DPC) 2” thick, Frame base block height 30cmx 30cm x 60cm CC block(1:2:4) at equal distance will be provided. The External & internal surface of the wall  will be plastered with 15mm thick cement mortar of Mix 1:4 ( 1 Cement :4 fine sand ) All plastered surface will be finished with two or more coat of samosam paint of approved brand.

Flooring /Drainage:

CC flooring made of crushed bricks 4” thick with 1 ½” graded stone (Rodi) which is further flushed with 4:1 (sand: cement), with proper drainage system.

Plinth Protection: 45cm wide with cement concrete at 1:2:4 ratio, 35 mm thickness, over a base of 75mm, thickness, made out in cement concrete with 40mm thickness brick ballast in ratio of 1:5:10, the work to including curing of the top surface and prefer compaction of the under surface.

Administrative Officer

NIPGR

Seal & Sign Of Contractor

TERMS & CONDITIONS

Name of Work: Supply & Installation of “Naturally Ventilated Poly House” at NIPGR Campus New 			   Delhi.

- The contractor should visit the site and make them self acquainted with the location of site. The Department shall not entertain any extra claim later on.

- The site of work is at NIPGR Campus, Aruna Asaf Ali Marg New Delhi-110067.

- The contractor shall make his own arrangement for the security of material at site.

- The contractor shall arrange all T&P and nothing extra will be paid.

- The contractor must get the prior approval of materials from the Engineer- In –Charge.

- The rates shall be inclusive of Transportation, loading, unloading and handling charges of material.

- The rates will be for finished items inclusive of the cost of material & Labour.

- Water & Electricity if required shall be provided by the Institute free of Cost.

- The rates shall be inclusive of all taxes including VAT & Service tax.

- The contractor shall follow all labour laws as prevailing in Delhi.

- All works shall be completed to the entire satisfaction of the Engineer-in-charge.

- The management reserves the right to reject / accept any or all quotation without assigning any reason.

- In case of any dispute the decision of the Engineer-in-charge shall be final & binding on the contractor.

- Any work not carried out as per instructions of the Engineer-in-charge shall be dismantled and re-constructed by the contractor without any extra cost.

- The work shall be completed in all respects within Four weeks after issue of issue of work order.  Being a time bound contract, and the contractor shall be liable to pay compensation @1.0% per day subject to maximum of 10% of work order amount if the work is not completed within stipulated time of Completion.

- The Contractor shall be responsible for any damages and site clearance and nothing extra will be paid.  In case the site clearance and damages are not attended, a recovery amounting to Rs. 5,000/- (Rs. Five thousand only) shall be made.

- Payment shall be made after completion of work.

- The quotation must be accompanied with EMD of Rs. 4800.00(Rs. Four Thousand & eight hundred only) EMD shall be in the form of DD of scheduled bank drawn in favour of “Director NIPGR” New Delhi.

- The EMD shall be enclosed in a separate envelop duly marked EMD which shall be opened first.

- Security deposit @ 10% shall be deducted from the bill and shall be refunded after the completion of defect liability period of twelve Months. The EMD will however be adjusted towards security deposit.

- PAN & TIN issued by the respective departments must be mentioned while quoting the rates. The quotation without these numbers will not be considered.

- The contractor shall write the rates & Amount in word & figures both.

- All papers attached with this Quotation should be signed & returned in original.

Administrative Officer

PAN No. -------------------                                                                                                                      NIPGR

TIN No. --------------------

Seal & Sign Of Contractor